DELHI SKILL AND ENTERPRENEURSHIP UNIVERSITY

SESSION 2021-22

SUBJECT:-

COMPUTER  ORGANIZATION AND ARCHITECTURE

PRACTICAL FILE

COURSE :- BCA

ROLL NO :- 41221139

SUBMITTED BY :-							SUBMITTED TO:-

Sachin Rajbhar						        APURVA VASHIST

| INDEX |  |
| --- | --- |
| LOGIC GATES |  |
| PRACTICAL-1 | PRACTICAL-2 |
| Not, And & Or Gates | Nand, Nor, Ex-Or & Ex-nor Gates |
| ADDERS |  |
| PRACTICAL -3 | PRACTICAL-4 |
| Half Adder | Full Adder |
| KARNAUGH MAPS |  |
| PRACTICAL-5 | PRACTICAL -6 |
| Half Adder K-Map | Full Adder K-Map |
| Half Subtractor K-Map | Full Subtractor K-Map |
| FLIP-FLOP |  |
| PRACTICAL-7 | PRACTICAL-8 |
| SR Flip-Flop | JK Flip-Flop |
| SHIFT REGISTERS |  |
| PRACTICAL-9 | PRACTICAL-10 |
| Serial In-Parallel Out | Serial In-Serial Out |
| PRACTICAL-11 | PRACTICAL-12 |
| Multiplexer | Demultiplexer |
| PRACTICAL-13 |  |
| 3X8 Decoder |  |

PRACTICAL 1

AIM: To implement basic Logic Gates (NOT, AND, OR)

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

- NOT GATE

The NOT gate is a single input single output gate. This gate is also known as Inverter because it performs the inversion of the applied binary signal, i.e., it converts 0 into 1 or 1 into 0.

| Input (A) | Output (B) |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

Expression: B= A

- AND GATE

A Boolean operator which gives the value 1 if and only if all the operands are 1, and otherwise has a value of zero.

| Inputs | Outputs |
| --- | --- |

| A | B | C |
| --- | --- | --- |
| 1 | 0 | 0 |
| 0 | 1 | 0 |
| 0 | 0 | 0 |
| 1 | 1 | 1 |

Expression: C= A.B

- OR GATE

A logical operation which gives the value 1 if at least one operand has the value 1, and otherwise gives a value of zero.

| Inputs | Outputs |
| --- | --- |

| A | B | C |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |
| 0 | 0 | 0 |

Expression: C= A+B

NOT GATE DIAGRAM

3

AND GATE DIAGRAM

OR GATE DIAGRAM

PRACTICAL 2

AIM: To implement basic Logic Gates (NAND, NOR)

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

- NAND GATE

A Boolean operator which gives the value 0 if and only if all the operands have a value of 1, and otherwise has a value of one (equivalent to NOT AND).

| Inputs | Outputs |
| --- | --- |

| A | B | C |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

Expression: C= A.B

- NOR GATE

A Boolean operator which gives the value 1 if and only if all operands have a value of 0 and otherwise has a value of zero.

| Inputs | Outputs |
| --- | --- |

| A | B | C |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

Expression: C= A+B

- EX-OR GATE

The EXOR gate has a high output only when an odd number of inputs are high. For the two input EXOR gates, the output will be high when the set of input is either 01 or 10.

| Inputs | Outputs |
| --- | --- |

| A | B | C |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

Expression: C= A+B

4.EX- NOR GATE

The output of EX-NOR gate is 1 when both the inputs are 1 (high) and if both the inputs are 0 (low). If either one of the input is 1 (high) the output will be 0 (low). In other words, both the inputs should be in the same logic level for the output to be 1.

| Inputs | Outputs |
| --- | --- |

| A | B | C |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

Expression: C= A+B

NAND GATE DIAGRAM:

NOR GATE DIAGRAM:

EX- OR GATE DIAGRAM:

EX-NOR GATE DIAGRAM:

PRACTICAL 3

AIM: To implement HALF ADDER

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

HALF ADDER

Half Adder is a combinational logic circuit which is designed by connecting one EX-OR gate and one AND gate. The half adder circuit has two inputs: A and B, which add two input digits and generates a carry and a sum.

| Inputs | Outputs |
| --- | --- |

| A | B | Sum | Carry |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Expression:

Sum = A+B

Carry = A.B

HALF ADDER DIAGRAM:

PRACTICAL 4

AIM: To implement FULL ADDER

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

FULL ADDER

Full Adder is the adder which adds three inputs and produces two outputs which consists of two EX-OR gates, two AND gates and one OR gate.

| Inputs | Outputs |
| --- | --- |

| A | B | Cin | Sum | Carry |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Expression:

Sum= C+(A+B)

Carry=AB+C.(A+B)

FULL ADDER DIAGRAM:

PRACTICAL 5

AIM: To implement HALF SUBTRACTOR

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

HALF SUBTRACTOR

The Half-Subtractor is a combinational circuit which is used to perform subtraction of two bits. It has two inputs, A (minuend) and B (subtrahend) and two outputs Difference and Borrow. The logic symbol and truth table are shown below.

| Inputs | Outputs |
| --- | --- |

| A | B | Difference | Borrow |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |

Expression:

Sum = A+B Carry = A.B

HALF SUBTRACTOR DIAGRAM:

PRACTICAL 6

AIM: To implement FULL SUBTRACTOR

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

FULL SUBTRACTOR

The full subtractor is a combinational circuit which is used to perform subtraction of three input bits: the minuend, subtrahend, and borrow in The full subtractor generates two output bits: the difference and borrow out .

| Inputs | Outputs |
| --- | --- |

| A | B | Bin | Difference | Borrow |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Expression:

Sum=C⨁(A⨁B)

Carry=A'B+C.(A'+B)

FULL SUBTRACTOR DIAGRAM:

PRACTICAL 7

AIM: To implement SR FLIP/FLOP

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

SR FLIP/FLOP

SR flip flop, also known as SR latch is the basic and simplest type of flip flop. It is a single bit storage element. It has only two logic gates. The output of each gate is connected to the input of another gate. The SR flip flop can be constructed by using NAND gates or NOR gates.

| S | R | Q | Q’ | State |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | NO CHANGE |
| 0 | 0 | 1 | 1 |  |
| 0 | 1 | 0 | 0 | RESET Q>>0 |
| 0 | 1 | 1 | 0 |  |
| 1 | 0 | 0 | 1 | SET Q>>1 |
| 1 | 0 | 1 | 1 |  |
| 1 | 1 | 0 | X | INVALID |
| 1 | 1 | 1 | X |  |

Characteristic Expression:

Qn+1 = S + R Qn

SR FLIP FLOP DIAGRAM:

PRACTICAL 8

AIM: To implement JK FLIP FLOP

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

JK FLIP FLOP

A JK flip-flop is a sequential bi-state single-bit memory device named after its inventor by Jack Kil. In general it has one clock input pin (CLK), two data input pins (J and K), and two output pins (Q and Q̅).

| K | J | Q | Q’ | State |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | NO CHANGE |
| 0 | 0 | 1 | 1 |  |
| 0 | 1 | 0 | 0 | RESET Q>>0 |
| 0 | 1 | 1 | 0 |  |
| 1 | 0 | 0 | 1 | SET Q>>1 |
| 1 | 0 | 1 | 1 |  |
| 1 | 1 | 0 | 1 | TOGGLE |
| 1 | 1 | 1 | 0 |  |

Characteristic Expression:

Qn+1 = TQn + TQn

JK FLIP FLOP DIAGRAM:

PRACTICAL 9

AIM: To implement SIPO (Serial In Parallel Out) Register

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

SIPO (Serial In Parallel out)

The shift register, which allows serial input (one bit after the other through a single data line) and produces a parallel output is known as Serial-In Parallel-Out shift register.

Working:

Input is 110 i.e. 3 bits

| Clock | I/P | Q2 | Q1 | Q0 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 | 1 | 1 | 0 | 0 |
| 3 | 1 | 1 | 1 | 0 |

SIPO (Serial In Parallel out) DIAGRAM:

PRACTICAL 10

AIM: To implement SISO (Serial In Serial Out) Register

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

SISO (Serial In Serial out)

The shift register, which allows serial input (one bit after the other through a single data line) and produces a serial output is known as Serial-In Serial-Out shift register. Since there is only one output, the data leaves the shift register one bit at a time in a serial pattern, thus the name Serial-In Serial-Out Shift Register.

Working:

Input is 110 i.e. 3bits

| Clock | I/P | Q2 | Q1 | Q0 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 |
| 2 | 1 | 1 | 0 | 0 |
| 3 | 1 | 1 | 1 | 0 |
| 4 | _ | _ | 1 | 1 |
| 5 | _ | _ | _ | 1 |

SISO (Serial In Serial out) DIAGRAM:

PRACTICAL 11

AIM: To implement Multiplexer using logic gates

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

Multiplexer (using logic gates)

A digital multiplexer is similar to a multi-position switch with many inputs and only one output. It has control inputs to select a particular input. It is also called as a channel selector and abbreviated as Mux.

It is easier to build multiplexers using gates (small scale integration -SSI ICs) for a few select lines. But as the number of select lines increases, use of medium scale integration MSI ICs becomes the best choice.

A multiplexer can be designed using: a. basic logic gates b. universal gates (NAND & NOR).

Truth Table:

| S0 | S1 | Output |
| --- | --- | --- |
| 0 | 0 | I0 |
| 0 | 1 | I1 |
| 1 | 0 | I2 |
| 1 | 1 | I3 |

Multiplexer (using logic gates) DIAGRAM:

PRACTICAL 12

AIM: To implement De-Multiplexer using logic gates

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

De-Multiplexer (using logic gates)

A demultiplexer (also known as a demux or data distributor) is defined as a circuit that can distribute or deliver multiple outputs from a single input. A demultiplexer can perform as a single input with many output switches.

The demultiplexer’s output lines are ‘n’ in number, the select line number is ‘m’ and n = 2m. The control signal or select input code decides the output line to which the input has to be transmitted.

Truth Table:

| Input | Select Lines |  | Output lines |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| I | S1 | S0 | D0 | D1 | D2 | D3 |
| I | 0 | 0 | 1 | 0 | 0 | 0 |
| I | 0 | 1 | 0 | 1 | 0 | 0 |
| I | 1 | 0 | 0 | 0 | 1 | 0 |
| I | 1 | 1 | 0 | 0 | 0 | 1 |

DEMULTIPLEXER DIAGRAM:

D0, D1, D2, D3 comes as Output 0,0,1,0 respectively, when input data is 1 only, with selection line values 1 and 0.

PRACTICAL 13

AIM: To implement 3X8 Decoder using logic gates

APPARATUS: https://circuitverse.org/simulator

PROCEDURE:

Decorder (using logic gates)

A binary decoder (sometimes referred to as just a “decoder”) is a combinational logic circuit that converts binary information from the n coded inputs to a maximum of 2n unique outputs. Binary decoders are used in an extensive number of applications—notably data multiplexing and data demultiplexing and seven segment displays.

3X8 Decoder Truth Table:

| Inputs |  |  |  | Outputs |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D0 |  | D1 | D2 | D3 | D4 | D5 | D6 | D7 |
| 0 | 0 | 0 | 1 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |  | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 |  | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |  | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 |  | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |  | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

3X8 DECODER DIAGRAM: